Specification

E/One Sentry Advisor

Remote Monitoring

Panel Option

Basic Package

OPTION ON SIMPLEX AND DUPLEX NON PROTECT PLUS PANELS – BASIC PACKAGE

(OPTIONAL) Wireless Cellular Based Communication Package – The E/One Sentry Advisor uses a cellular transmitter to communicate trouble conditions and high level alarm. Each panel shall be equipped with hardware that is connected to dry contacts for high level alarm and pump running events. This information shall then be transmitted to a host system that utilizes an interactive website interface for end users to access the information.

A secure user ID and password shall be used to provide access to the website. The website shall quickly identify sites that may be experiencing trouble, and shall be able to generate reports including: running tallies of usage, flow rates, potential infiltration, predictive maintenance indicators and asset mapping/management. The website shall also provide a means to configure alerts corresponding to the various trouble conditions that can be sent to users via secure e-mail, SMS text, phone or an existing telemetry system. These alerts can be sent to any number of recipients.

Accessible data shall include:

- Pump Runtime

- Pump Cycles

- Min, Max, Avg. and Last Runtime

- Trouble Events

  - High Level Alarm

  - Excessive Runtime

- Average GPM based Flow Rate Data

The E/One Sentry Advisor shall be equipped with a battery that will allow one message to be sent out in the event of a power loss.

The communication package shall be a factory prewired unit within the panel enclosure including the compact cellular transmitter, power supply and antenna. Panels with this option must be within range of the associated cellular network.

Environment One Corporation

2773 Balltown Road

Niskayuna, New York 12309

www.eone.com

A Precision Castparts Company

LM000422 Rev A